Custom Ti Abutments – Design Only

Must be sent to Argon Dental USA’s certified milling center for manufacturing:
https://www.argondentalusa.com/stl-submission-form/

Gingiva Height:
0.5 mm		        1.5 mm

2mm (Red Platform)
K3 Pro implants with 3.0 or 3.5 mm diameters, 9.0 – 17.0 mm long, and 2.0 mm platform

Part Numbers:

CAD.UNI 100005/2Pro
	CAD.UNI 100015/2Pro

Gingiva Heights: 0.5 & 1.5 mm

Blank Diameter: 10 mm

Scan Bodies: Tri-Scan K (CGSB_K3Pro/2_k or CGSB_K3Pro/2.PK_k)

3Shape Selection

- Select “Abutment” and “Customized Abutment”

- Category – Select “Argon K3Pro (Argon Dental USA)”

- System – Select “Argon K3Pro CAD.UNI (Custom Abutment)”

- Kit – Select “CAD.UNI 2.0mm X.5GH”

Custom Ti Abutments – Design Only

Must be sent to Argon Dental USA’s certified milling center for manufacturing:
https://www.argondentalusa.com/stl-submission-form/

3mm (Yellow Platform) & 3mm (Blue Platform)
K3 Pro implants with 4.0 - 6.0 mm diameters, 8.0 - 17.0 mm long, and 3.0 mm platform 
K3 Pro implants with 4.0 mm diameters, 5.5 or 6.5 mm long, and 3.0 mm platform

Gingiva Height:
0.5 mm		        1.5 mm

Part Numbers:

CAD.UNI 100005/3Pro
	CAD.UNI 100015/3Pro

Gingiva Heights: 0.5 & 1.5 mm

Blank Diameter: 10 mm

Scan Bodies: Tri-Scan K (CGSB_K3Pro/3_k or CGSB_K3Pro/3.PK_k)

Note: For 3mm short blue platform the corresponding blue
           screw needs to be used

3Shape Selection

- Select “Abutment” and “Customized Abutment”

- Category – Select “Argon K3Pro (Argon Dental USA)”

- System – Select “Argon K3Pro CAD.UNI (Custom Abutment)”

- Kit – Select “CAD.UNI 3.0mm X.5GH”

Crowns & Ti-Base Zirconium Abutments – KSA – In Lab Manufacturing

2mm (Red Platform)
K3 Pro implants with 3.0 or 3.5 mm diameters, 9.0 – 17.0 mm long, and 2.0 mm platform

Part Numbers:

|  | 0° | 10° | 20° |
| --- | --- | --- | --- |
| 0.5 mm | KSA400005.H/2Pro | KSA401005.H/2Pro | KSA402005.H/2Pro |
| 1.5 mm | KSA400015.H/2Pro | KSA401015.H/2Pro | KSA402015.H/2Pro |
| 2.5 mm | KSA400025.H/2Pro | KSA401025.H/2Pro | KSA402025.H/2Pro |

Gingiva Heights: 0.5, 1.5, 2.5 mm

Diameter: 4.1 mm

Angulation: 0°, 10°, & 20°

Scan Bodies: Tri-Scan K (CGSB_K3Pro/2_k or CGSB_K3Pro/2.PK_k) 
                        or Scan Cap (KSA_K4.0 or KSA_PT4.0)

3Shape Selection

- Select “Abutment” and “Customized Abutment ”, 
“Anatomical Abutment”, or “Screw Retained Crown"

- Category– Select “Argon K3pro (Argon Dental USA)”

- System – Select “Argon K3 Pro KSA (Ti-Base)”

- Kit – Select “KSA 2.0mm X0° X.XGH”

Crowns & Ti-Base Zirconium Abutments – KSA – In Lab Manufacturing

3mm (Yellow Platform) & 3mm (Blue Platform)
K3 Pro implants with 4.0 - 6.0 mm diameters, 8.0 - 17.0 mm long, and 3.0 mm platform 
K3 Pro implants with 4.0 mm diameters, 5.5 or 6.5 mm long, and 3.0 mm platform

Part Numbers:

|  | 0° | 10° | 20° |
| --- | --- | --- | --- |
| 0.5 mm | KSA500005.H/2Pro | KSA501005.H/2Pro | KSA502005.H/2Pro |
| 1.5 mm | KSA500015.H/2Pro | KSA501015.H/2Pro | KSA502015.H/2Pro |
| 2.5 mm | KSA500025.H/2Pro | KSA501025.H/2Pro | KSA502025.H/2Pro |

Gingiva Heights: 0.5, 1.5, 2.5 mm

Diameter: 4.3 mm

Angulation: 0°, 10°, & 20°

Scan Bodies: Tri-Scan K (CGSB_K3Pro/3_k or CGSB_K3Pro/3.PK_k)
                        or Scan Cap (KSA_K4.0 or KSA_PT4.0)

Note: For 3mm short blue platform the corresponding blue screw needs 
           to be used

3Shape Selection

- Select “Abutment” and “Customized Abutment ”, 
“Anatomical Abutment”, or “Screw Retained Crown"

- Category– Select “Argon K3pro (Argon Dental USA)”

- System – Select “Argon K3 Pro KSA (Ti-Base)”

- Kit – Select “KSA 3.0mm X0° X.XGH”

Crowns & Ti-Base Zirconium Abutments – Flexi-Base (CG.V) – In Lab Manufacturing

2mm (Red Platform)
K3 Pro implants with 3.0 or 3.5 mm diameters, 9.0 – 17.0 mm long, and 2.0 mm platform

Gingiva Height:
0.5 mm		              1.5 mm

Part Numbers:

CG.V0005.H/2Pro.tc					CG.V0015.H/2Pro.tc

Gingiva Heights: 0.5 & 1.5 mm

Length: 10 mm (adjustable to a minimum of 4 mm)

Diameter: 4.3 mm

Angulation: 0°

Scan Bodies: Tri-Scan K (CGSB_K3Pro/2_k 
                         or CGSB_K3Pro/2.PK_k)

3Shape Selection

- Select “Abutment” and “Customized Abutment ”, 
“Anatomical Abutment”, or “Screw Retained Crown"

- Category– Select “Argon K3pro (Argon Dental USA)”

- System – Select “Argon K3 Pro CG.V (Flexi-Base)”

- Kit – Select “Flexi 2.0mm X.XGH”

Crowns & Ti-Base Zirconium Abutments – Flexi-Base (CG.V) – In Lab Manufacturing

3mm (Yellow Platform) & 3mm (Blue Platform)
K3 Pro implants with 4.0 - 6.0 mm diameters, 8.0 - 17.0 mm long, and 3.0 mm platform 
K3 Pro implants with 4.0 mm diameters, 5.5 or 6.5 mm long, and 3.0 mm platform

Gingiva Height:
0.5 mm		              1.5 mm

Part Numbers:

CG.V0005.H/3Pro.tc					CG.V0015.H/3Pro.tc

Gingiva Heights: 0.5 & 1.5 mm

Length: 10 mm (adjustable to a minimum of 4 mm)

Diameter: 4.3 mm

Angulation: 0°

Scan Bodies: Tri-Scan (CGSB_K3Pro/3_k 
                        or CGSB_K3Pro/3.PK_k)

Note: For 3mm short blue platform the corresponding 
           blue screw needs to be used

3Shape Selection

- Select “Abutment” and “Customized Abutment ”, 
“Anatomical Abutment”, or “Screw Retained Crown"

- Category– Select “Argon K3pro (Argon Dental USA)”

- System – Select “Argon K3 Pro CG.V (Flexi-Base)”

- Kit – Select “Flexi 3.0mm X.XGH”

Crowns & Ti-Base Zirconium Abutments – Clou-Base CG.C – In Lab Manufacturing

2mm (Red Platform)
K3 Pro implants with 3.0 or 3.5 mm diameters, 9.0 – 17.0 mm long, and 2.0 mm platform

Gingiva Height:
F1 1.5 mm           	   P2 1.5 mm

|  | F1 (Front) | P2 (Incisors) |
| --- | --- | --- |
| 1.5 mm | CG.C0015.H/2Pro.F1.tc | CG.C0015.H/2Pro.P2.tc |

Part Numbers:

Gingiva Heights: 0.5 & 1.5 mm

Scan Bodies: Tri-Scan (CGSB_K3Pro/2_k or 
                        GSB_K3Pro/2.PK_k)

3Shape Selection

- Select “Abutment” and “Customized Abutment ”, 
“Anatomical Abutment”, or “Screw Retained Crown"

- Category– Select “Argon K3pro (Argon Dental USA)”

- System – Select “Argon K3 Pro CG.C (Clou-Base)”

- Kit – Select “Clou 2.0mm XX X.XGH”

Crowns & Ti-Base Zirconium Abutments – Clou-Base CG.C – In Lab Manufacturing

3mm (Yellow Platform) & 3mm (Blue Platform)
K3 Pro implants with 4.0 - 6.0 mm diameters, 8.0 - 17.0 mm long, and 3.0 mm platform 
K3 Pro implants with 4.0 mm diameters, 5.5 or 6.5 mm long, and 3.0 mm platform

Gingiva Height:
0.5 mm		              1.5 mm

|  | P1 (Incisors) | P2 (Pre-molars) | M1 (Molars) |
| --- | --- | --- | --- |
| 0.5 mm | CG.C0005.H/3Pro.P1.tc | CG.C0005.H/3Pro.P2.tc | CG.C0005.H/3Pro.M1.tc |
| 1.5 mm | CG.C0015.H/3Pro.P1.tc | CG.C0015.H/3Pro.P2.tc | CG.C0015.H/3Pro.M1.tc |

Part Numbers:

Gingiva Heights: 0.5 & 1.5 mm

Scan Bodies: Tri-Scan (CGSB_K3Pro/3_k or CGSB_K3Pro/3.PK_k)

Note: For 3mm short blue platform the corresponding blue screw needs to be used

3Shape Selection

- Select “Abutment” and “Customized Abutment ”, 
“Anatomical Abutment”, or “Screw Retained Crown"

- Category– Select “Argon K3pro (Argon Dental USA)”

- System – Select “Argon K3 Pro CG.C (Clou-Base)”

- Kit – Select “Clou 3.0mm XX X.XGH”